RECORDER’S STANDARDS

(Amended 07/11/2025)

In order to provide legible and reproducible recorded maps, in coordinated with the County Recorder in establishing the following requirements.  These requirements are in addition to the State Subdivision Map Act Requirement, Land Surveyors Act or any other State statute that governs the filing of maps with the County Recorder.

- Maps must be drawn with black permanent ink (not gray).

- Maps must be drawn on the top of the Mylar (not the back side).

- Maps must be drawn on white Mylar (Polyester)(No off-color) 3 mil or 4 mil thick.

- Adequate lettering (not thin).  Smallest letters and numbers not less than 8 pt.

- Signature must not obscure printed text. (See example below.)

- No shading. (Shading does not show up on scanning or microphotography.)

- Line width and character stroke width must be between .014" and .020".

- Character width must be 80%-100% (do not use condensed or compressed at a rate of less than 80%).

- If an original is submitted in which portions of the map have been duplicated it must be a legible direct positive Mylar.

- Street names must be clearly shown.

- Map size is to be 18” x 26” with a 1” blank border.

- Maps must have the San Bernardino County Recorder’s Certificate printed on them prior to submission for recordation.  The certificate shall appear in the lower or upper right hand corner on the first map sheet.

Effective July 7, 2025, all maps submitted for recording must follow these requirements.  Failure to follow these requirements may adversely affect the recording process of your map.

Signature must not obscure printed text.