| Form 4.2-5 HCOC Assessment for Peak Runoff (DA ) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compute peak runoff for pre and post developed conditions |  |  |  |  |  |  |  |  |  |
| Variables |  |  | Pre-developed DA to Project Outlet Use additional forms if more than 3 DMA |  |  |  | Post-developed DA to Project Outlet Use additional forms if more than 3 DMA |  |  |
|  |  |  | DMA A | DMA B |  | DMA C | DMA A | DMA B | DMA C |
| 1 Rainfall Intensity for storm duration equal to time of concentration Ipeak = 10^(LOG Form 4.2-1 Item 4 - 0.6 LOG Form 4.2-4 Item 5 /60) |  |  |  |  |  |  |  |  |  |
| 2 Drainage Area of each DMA (ft2) For DMA with outlet at project site outlet, include upstream DMA (Using example schematic in Form 3-1, DMA A will include drainage from DMA C) |  |  |  |  |  |  |  |  |  |
| 3 Ratio of pervious area to total area For DMA with outlet at project site outlet, include upstream DMA (Using example schematic in Form 3-1, DMA A will include drainage from DMA C) |  |  |  |  |  |  |  |  |  |
| 4 Pervious area infiltration rate (in/hr) Use pervious area CN and antecedent moisture condition with Appendix C-3 of the TGD for WQMP |  |  |  |  |  |  |  |  |  |
| 5 Maximum loss rate (in/hr) Fm = Item 3 * Item 4 Use area-weighted Fm from DMA with outlet at project site outlet, include upstream DMA (Using example schematic in Form 3-1, DMA A will include drainage from DMA C) |  |  |  |  |  |  |  |  |  |
| 6 Peak Flow from DMA (cfs) Qp =Item 2 * 0.9 * (Item 1 - Item 5) |  |  |  |  |  |  |  |  |  |
| 7 Time of concentration adjustment factor for other DMA to site discharge point Form 4.2-4 Item 12 DMA / Other DMA upstream of site discharge point (If ratio is greater than 1.0, then use maximum value of 1.0) |  | DMA A | n/a |  |  |  | n/a |  |  |
|  |  | DMA B |  | n/a |  |  |  | n/a |  |
|  |  | DMA C |  |  |  | n/a |  |  | n/a |
| 8 Pre-developed Qp at Tc for DMA A: Qp = Item 6DMAA + [Item 6DMAB * (Item 1DMAA - Item 5DMAB)/(Item 1DMAB - Item 5DMAB)* Item 7DMAA/2] + [Item 6DMAC * (Item 1DMAA - Item 5DMAC)/(Item 1DMAC - Item 5DMAC)* Item 7DMAA/3] | 9 Pre-developed Qp at Tc for DMA B: Qp = Item 6DMAB + [Item 6DMAA * (Item 1DMAB - Item 5DMAA)/(Item 1DMAA - Item 5DMAA)* Item 7DMAB/1] + [Item 6DMAC * (Item 1DMAB - Item 5DMAC)/(Item 1DMAC - Item 5DMAC)* Item 7DMAB/3] |  |  |  | 10 Pre-developed Qp at Tc for DMA C: Qp = Item 6DMAC + [Item 6DMAA * (Item 1DMAC - Item 5DMAA)/(Item 1DMAA - Item 5DMAA)* Item 7DMAC/1] + [Item 6DMAB * (Item 1DMAC - Item 5DMAB)/(Item 1DMAB - Item 5DMAB)* Item 7DMAC/2] |  |  |  |  |
| 10 Peak runoff from pre-developed condition confluence analysis (cfs): Maximum of Item 8, 9, and 10 (including additional forms as needed) |  |  |  |  |  |  |  |  |  |
| 11 Post-developed Qp at Tc for DMA A: Same as Item 8 for post-developed values | 12 Post-developed Qp at Tc for DMA B: Same as Item 9 for post-developed values |  |  |  | 13 Post-developed Qp at Tc for DMA C: Same as Item 10 for post-developed values |  |  |  |  |
| 14 Peak runoff from post-developed condition confluence analysis (cfs): Maximum of Item 11, 12, and 13 (including additional forms as needed) |  |  |  |  |  |  |  |  |  |
| 15 Peak runoff reduction needed to meet HCOC Requirement (cfs): Qp-HCOC = (Item 14 * 0.95) – Item 10 |  |  |  |  |  |  |  |  |  |

| Form 4.2-5 HCOC Assessment for Peak Runoff (DA ) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compute peak runoff for pre and post developed conditions |  |  |  |  |  |  |  |  |  |
| Variables |  |  | Pre-developed DA to Project Outlet |  |  |  | Post-developed DA to Project Outlet |  |  |
|  |  |  | DMA | DMA |  | DMA | DMA | DMA | DMA |
| 1 Rainfall Intensity for storm duration equal to time of concentration Ipeak = 10^(LOG Form 4.2-1 Item 4 - 0.6 LOG Form 4.2-4 Item 5 /60) |  |  |  |  |  |  |  |  |  |
| 2 Drainage Area of each DMA (ft2) For DMA with outlet at project site outlet, include upstream DMA (Using example schematic in Form 3-1, DMA A will include drainage from DMA C) |  |  |  |  |  |  |  |  |  |
| 3 Ratio of pervious area to total area For DMA with outlet at project site outlet, include upstream DMA (Using example schematic in Form 3-1, DMA A will include drainage from DMA C) |  |  |  |  |  |  |  |  |  |
| 4 Pervious area infiltration rate (in/hr) Use pervious area CN and antecedent moisture condition with Appendix C-3 of the TGD for WQMP |  |  |  |  |  |  |  |  |  |
| 5 Maximum loss rate (in/hr) Fm = Item 3 * Item 4 Use area-weighted Fm from DMA with outlet at project site outlet, include upstream DMA (Using example schematic in Form 3-1, DMA A will include drainage from DMA C) |  |  |  |  |  |  |  |  |  |
| 6 Peak Flow from DMA (cfs) Qp =Item 2 * 0.9 * (Item 1 - Item 5) |  |  |  |  |  |  |  |  |  |
| 7 Time of concentration adjustment factor for other DMA to site discharge point Form 4.2-4 Item 12 DMA / Other DMA upstream of site discharge point (If ratio is greater than 1.0, then use maximum value of 1.0) |  | DMA D | n/a |  |  |  | n/a |  |  |
|  |  | DMA E |  | n/a |  |  |  | n/a |  |
|  |  | DMA F |  |  |  | n/a |  |  | n/a |
| 8 Pre-developed Qp at Tc for DMA : Qp = Item 6DMAA + [Item 6DMAB * (Item 1DMAA - Item 5DMAB)/(Item 1DMAB - Item 5DMAB)* Item 7DMAA/2] + [Item 6DMAC * (Item 1DMAA - Item 5DMAC)/(Item 1DMAC - Item 5DMAC)* Item 7DMAA/3] | 9 Pre-developed Qp at Tc for DMA : Qp = Item 6DMAB + [Item 6DMAA * (Item 1DMAB - Item 5DMAA)/(Item 1DMAA - Item 5DMAA)* Item 7DMAB/1] + [Item 6DMAC * (Item 1DMAB - Item 5DMAC)/(Item 1DMAC - Item 5DMAC)* Item 7DMAB/3] |  |  |  | 10 Pre-developed Qp at Tc for DMA : Qp = Item 6DMAC + [Item 6DMAA * (Item 1DMAC - Item 5DMAA)/(Item 1DMAA - Item 5DMAA)* Item 7DMAC/1] + [Item 6DMAB * (Item 1DMAC - Item 5DMAB)/(Item 1DMAB - Item 5DMAB)* Item 7DMAC/2] |  |  |  |  |
| 10 Reserved |  |  |  |  |  |  |  |  |  |
| 11 Post-developed Qp at Tc for DMA : Same as Item 8 for post-developed values | 12 Post-developed Qp at Tc for DMA : Same as Item 9 for post-developed values |  |  |  | 13 Post-developed Qp at Tc for DMA : Same as Item 10 for post-developed values |  |  |  |  |
| 14 Reserved |  |  |  |  |  |  |  |  |  |
| 15 Reserved |  |  |  |  |  |  |  |  |  |