| Form 4.3-10 Hydromodification Control BMPs (DA ) |  |  |
| --- | --- | --- |
| 1 Volume reduction needed for HCOC performance criteria (ft3): (Form 4.2-2 Item 4 * 0.95) – Form 4.2-2 Item 1 |  | 2 On-site retention with site design hydrologic source control, infiltration, and harvest and use LID BMP (ft3): Sum of Form 4.3-9 Items 2, 3, and 4 Evaluate option to increase implementation of on-site retention in Forms 4.3-2, 4.3-3, and 4.3-4 in excess of LID DCV toward achieving HCOC volume reduction |
| 3 Remaining volume for HCOC volume capture (ft3): Item 1 – Item 2 | 4 Volume capture provided by incorporating additional on-site or off-site retention BMPs (ft3): Existing downstream BMP may be used to demonstrate additional volume capture (if so, attach to this WQMP a hydrologic analysis showing how the additional volume would be retained during a 2-yr storm event for the regional watershed) |  |
| 5 If Item 4 is less than Item 3, incorporate in-stream controls on downstream waterbody segment to prevent impacts due to hydromodification Attach in-stream control BMP selection and evaluation to this WQMP |  |  |
| 6 Is Form 4.2-2 Item 11 less than or equal to 5%: Yes No If yes, HCOC performance criteria is achieved. If no, select one or more mitigation options below: - Demonstrate increase in time of concentration achieved by proposed LID site design, LID BMP, and additional on-site or off-site retention BMP BMP upstream of a waterbody segment with a potential HCOC may be used to demonstrate increased time of concentration through hydrograph attenuation (if so, show that the hydraulic residence time provided in BMP for a 2-year storm event is equal or greater than the addition time of concentration requirement in Form 4.2-4 Item 15) - Increase time of concentration by preserving pre-developed flow path and/or increase travel time by reducing slope and increasing cross-sectional area and roughness for proposed on-site conveyance facilities - Incorporate appropriate in-stream controls for downstream waterbody segment to prevent impacts due to hydromodification, in a plan approved and signed by a licensed engineer in the State of California |  |  |
| 7 Form 4.2-2 Item 12 less than or equal to 5%: Yes No If yes, HCOC performance criteria is achieved. If no, select one or more mitigation options below: - Demonstrate reduction in peak runoff achieved by proposed LID site design, LID BMPs, and additional on-site or off-site retention BMPs BMPs upstream of a waterbody segment with a potential HCOC may be used to demonstrate additional peak runoff reduction through hydrograph attenuation (if so, attach to this WQMP, a hydrograph analysis showing how the peak runoff would be reduced during a 2-yr storm event) - Incorporate appropriate in-stream controls for downstream waterbody segment to prevent impacts due to hydromodification, in a plan approved and signed by a licensed engineer in the State of California |  |  |