| Form 4.3-2 Site Design Hydrologic Source Control BMPs (DA ) |  |  |  |
| --- | --- | --- | --- |
| 1 Implementation of Impervious Area Dispersion BMP (i.e. routing runoff from impervious to pervious areas), excluding impervious areas planned for routing to on-lot infiltration BMP: Yes No If yes, complete Items 2-5; If no, proceed to Item 6 | DA DMA BMP Type | DA DMA BMP Type | DA DMA BMP Type (Use additional forms for more BMPs) |
| 2 Total impervious area draining to pervious area (ft2) |  |  |  |
| 3 Ratio of pervious area receiving runoff to impervious area |  |  |  |
| 4 Retention volume achieved from impervious area dispersion (ft3) V = Item2 * Item 3 * (0.5/12), assuming retention of 0.5 inches of runoff |  |  |  |
| 5 Sum of retention volume achieved from impervious area dispersion (ft3): Vretention =Sum of Item 4 for all BMPs |  |  |  |
| 6 Implementation of Localized On-lot Infiltration BMPs (e.g. on-lot rain gardens): Yes No If yes, complete Items 7-13 for aggregate of all on-lot infiltration BMP in each DA; If no, proceed to Item 14 | DA DMA BMP Type | DA DMA BMP Type | DA DMA BMP Type (Use additional forms for more BMPs) |
| 7 Ponding surface area (ft2) |  |  |  |
| 8 Ponding depth (ft) |  |  |  |
| 9 Surface area of amended soil/gravel (ft2) |  |  |  |
| 10 Average depth of amended soil/gravel (ft) |  |  |  |
| 11 Average porosity of amended soil/gravel |  |  |  |
| 12 Retention volume achieved from on-lot infiltration (ft3) Vretention = (Item 7 *Item 8) + (Item 9 * Item 10 * Item 11) |  |  |  |
| 13 Runoff volume retention from on-lot infiltration (ft3): Vretention =Sum of Item 12 for all BMPs |  |  |  |

| Form 4.3-2 cont. Site Design Hydrologic Source Control BMPs (DA ) |  |  |  |
| --- | --- | --- | --- |
| 14 Implementation of evapotranspiration BMP (green, brown, or blue roofs): Yes No If yes, complete Items 15-20. If no, proceed to Item 21 | DA DMA BMP Type | DA DMA BMP Type | DA DMA BMP Type (Use additional forms for more BMPs) |
| 15 Rooftop area planned for ET BMP (ft2) |  |  |  |
| 16 Average wet season ET demand (in/day) Use local values, typical ~ 0.1 |  |  |  |
| 17 Daily ET demand (ft3/day) Item 15 * (Item 16 / 12) |  |  |  |
| 18 Drawdown time (hrs) Copy Item 6 in Form 4.2-1 |  |  |  |
| 19 Retention Volume (ft3) Vretention = Item 17 * (Item 18 / 24) |  |  |  |
| 20 Runoff volume retention from evapotranspiration BMPs (ft3): Vretention =Sum of Item 19 for all BMPs |  |  |  |
| 21 Implementation of Street Trees: Yes No If yes, complete Items 22-25. If no, proceed to Item 26 | DA DMA BMP Type | DA DMA BMP Type | DA DMA BMP Type (Use additional forms for more BMPs) |
| 22 Number of Street Trees |  |  |  |
| 23 Average canopy cover over impervious area (ft2) |  |  |  |
| 24 Runoff volume retention from street trees (ft3) Vretention = Item 22 * Item 23 * (0.05/12) assume runoff retention of 0.05 inches |  |  |  |
| 25 Runoff volume retention from street tree BMPs (ft3): Vretention = Sum of Item 24 for all BMPs |  |  |  |
| 26 Implementation of residential rain barrels/cisterns: Yes No If yes, complete Items 27-29; If no, proceed to Item 30 | DA DMA BMP Type | DA DMA BMP Type | DA DMA BMP Type (Use additional forms for more BMPs) |
| 27 Number of rain barrels/cisterns |  |  |  |
| 28 Runoff volume retention from rain barrels/cisterns (ft3) Vretention = Item 27 * 3 |  |  |  |
| 29 Runoff volume retention from residential rain barrels/Cisterns (ft3): Vretention =Sum of Item 28 for all BMPs |  |  |  |
| 30 Total Retention Volume from Site Design Hydrologic Source Control BMPs: Sum of Items 5, 13, 20, 25 and 29 |  |  |  |