| Table A-11 Bioretention (with Underdrain) |  |  |  |
| --- | --- | --- | --- |
| Variable Use columns to the right to compute runoff volume retention from Bioretention (w/o Underdrain) BMPs | DA | DA | DA |
| 1 Infiltration rate of underlying soils (in/hr) See Guidance Section 5.4.2 and Appendix D for minimum requirements for assessment methods. |  |  |  |
| 2 Infiltration safety factor See Guidance Section 5.4.2 and Appendix D |  |  |  |
| 3 Design percolation rate (in/hr) Pdesign = Item 1 / Item 2 |  |  |  |
| 4 Ponded water drawdown time (hr), default is 48 hrs |  |  |  |
| 5 Maximum ponding depth (ft) BMP specific, see Table 5-4 in Guidance for reference to BMP design details |  |  |  |
| 6 Ponding Depth (ft) dBMP = Minimum of (1/12 * Item 2 * Item 3) or Item 5 |  |  |  |
| 7 Infiltrating surface area, SABMP (ft2) area beneath gravel layer for BMPs without underdrains |  |  |  |
| 8 Amended soil depth, dmedia (ft) Only included in certain BMP types, see Table 5-4 in Guidance for reference to BMP design details |  |  |  |
| 9 Amended soil porosity |  |  |  |
| 10 Gravel depth, dmedia (ft) Only included in certain BMP types, see Table 5-4 in Guidance for reference to BMP design details |  |  |  |
| 11 Gravel porosity |  |  |  |
| 12 Duration of storm as basin is filling (hrs) Typical ~ 3hrs |  |  |  |
| 13 Retention Volume (ft3) Vretention = Item 7 * [Item 6 + (Item 8 * Item 9) + (Item 10 * Item 11) + (Item 12 * (Item 3 / 12))] |  |  |  |