| Table A-6 Drainage Swale |  |  |  |
| --- | --- | --- | --- |
| Variable Use columns to the right to compute runoff volume treatment from proposed Drainage Swales | DA | DA | DA |
| 1 Pollutants addressed with BMP List all pollutant of concern that will be effectively reduced through specific Unit Operations and Processes described in Table 5-5 of the WQMP Guidance |  |  |  |
| 2 Flow depth for water quality treatment (ft) BMP specific, see Table 5-6 in TGD for WQMP for reference to BMP design details |  |  |  |
| 3 Bed slope (ft/ft) BMP specific, see Table 5-6 in TGD for WQMP for reference to BMP design details |  |  |  |
| 4 Manning's roughness coefficient |  |  |  |
| 5 Bottom width (ft): bw = (Form 4.3-5 Item 6 * Item 4) / (1.49 * Item 2^1.67 * Item 3^0.5) |  |  |  |
| 6 Side Slope (ft/ft) BMP specific, see Table 5-6 in TGD for WQMP for reference to BMP design details |  |  |  |
| 7 Cross sectional area (ft2): A = (Item 5 * Item 2) + (Item 6 * Item 2^2) |  |  |  |
| 8 Water quality flow velocity (ft/sec): V = Form 4.3-5 Item 6 / Item 7 |  |  |  |
| 9 Flow capacity (cfs): Q = Item 7 * Item 8 |  |  |  |
| 10 Hydraulic residence time (min) Pollutant specific, see Table 5-6 in TGD for WQMP for reference to BMP design details |  |  |  |
| 11 Length of flow based BMP (ft): L = Item 8 * Item 10 * 60 |  |  |  |
| 12 Water surface area at water quality flow depth (ft2): SAtop = (Item 5 + (2 * Item 2 * Item 6)) * Item 11 |  |  |  |
| 13 LID BMP Volume equivalency (%): Use Item 9 ( flow capacity) and Figure 5-2 in the TGD for WQMP |  |  |  |