| Table A-7 Infiltration Basins |  |  |  |
| --- | --- | --- | --- |
| Variable Use columns to the right to compute runoff volume retention from Infiltration Basin and Infiltration Trench BMPs | DA | DA | DA |
| 1 Infiltration rate of underlying soils (in/hr), See Section 5.4.2 and Appendix D of the TGD for WQMP for minimum requirements for assessment methods. |  |  |  |
| 2 Infiltration safety factor, See Section 5.4.2 and Appendix D of the TGD for WQMP |  |  |  |
| 3 Design percolation rate (in/hr): Pdesign = Item 1 / Item 2 |  |  |  |
| 4 Infiltrating surface area, SABMP (ft2), surface area of basin or trench bottom |  |  |  |
| 5 Ponded water drawdown time (hr), default is 48 hrs |  |  |  |
| 6 Duration of storm as basin is filling (hrs) Typical ~ 3hrs |  |  |  |
| 7 Ponding surface area, SAponded (ft2), Only included in certain BMP types, see Table 5-4 in the TGD for WQMP for reference to BMP design details |  |  |  |
| 8 Ponding Depth (ft): dpond = Minimum of (1/12 * Item 3 * Item 5) or maximum ponding depth – see Section 5.4.2 and Appendix D of the TGD for WQMP for minimum requirements for assessment methods |  |  |  |
| 9 Gravel layer surface area, SAgravel (ft2), Only included in certain BMP types, see Table 5-4 in the TGD for WQMP for reference to BMP design details |  |  |  |
| 10 Gravel depth, dgravel (ft) Only included in certain BMP types, see Table 5-4 in the TGD for WQMP for reference to BMP design details |  |  |  |
| 11 Gravel porosity, Only included in certain BMP types, see Table 5-4 in the TGD for WQMP for reference to BMP design details |  |  |  |
| 12a Basin Retention Volume (ft3): Vretention = Item 3 *Item 4 * (Item 5 + Item 6) |  |  |  |
| 12b Trench Retention Volume (ft3): Vretention = (Item 3 * Item 4 * Item 6) + (Item 7 * Item 8) + (Item9 * Item 10 * Item 11) |  |  |  |